UHF All-In-One Reader

Specification

Product Model：CJ2503A

Date：        2016/07/24

Version Code： V1.0.0

1.  Product Feature

- Using Impinj R2000 RF Platform

- IP66 protection, adapt to work outside all weather

- CPU:ARM9 platform, basic frequency is 400MHz

- Support ISO18000-6B/6C dual protocol

- Built-in Linux 2.6 operating system, support customer’s customized and extendable development

- Support variety of communication interfaces, like RJ-45/RS-232/RS-485/Wiegand etc

- Built-in 9dBi circular polarization antenna

- Support one external antenna to achieve one driving two devices set up

2.  Product Introduction

CJ2503A is a high-performance UHF One-Piece Reader which integrated antenna and reader, and support ISO18000-6C/6B protocols. Its working frequency covers national standard dual frequency: 920MHz-925MHz, 840MHz-845MHz, FCC:902MHz-928MHz,ETSI:865MHz-868 MHz, and JP:916MHz-920MHz. Its output power is 0dBm-33dBm, and it has many good features, like long distance identification, fast speed, strong multi-label identification, strong anti-interference, high level protection and easy installing.

3.  Typical Applications

- Intelligent vehicle management, like vehicle access control, ETC, Customs  Clearance, Parking, Automatic Car Weighing Industry etc

- Automatic manufacturing, visual management etc

- Logistics Industry, like container management, pallet management

- Electronic Ticket, Card sender and Person Access control

- Asset warehousing management

4.  Main Technical Specification

| Physical Data |  |
| --- | --- |
| Size | 290mm×290mm×115mm |
| Weight | About 2.5kg(Not including accessory) |
| Shell material | Alloy+PC |
| Power | DC 24V/2.5A |
| Working Power consumption | 20W(Output Power 30dBm) |
| UHF RFID |  |
| Protocol | ISO18000-6B/6C |
| Frequency | GB :920～925MHz,840MHz～845MHz FCC: 902～928MHz ETSI: 865～868MHz JP :916～920MHz （Optional） |
| Output Power | 0～33dBm(adjustable by software) |
| Power Regulation | 1dB Step |
| Spectrum bandwidth | ＜200kHz |
| Spectrum Stability | ≤±10ppm |
| External Interface | RJ-45，RS232，RS485, Wiegand |
| Operating Mode | Fixed Frequency or Frequency hopping mode |
| RJ-45 communication rate | 10M/100M self-adaptive |
| RS-232 interface | Baud Rate 115200bps(Default) |
| I/O | 2 relay output/ 2 optical coupler input |
| Reading speed | >400 time/s |
| Antenna | Built-in 9dBi circular polarization antenna, and support on external antenna |
| Reading distance | 0～10m (According to the label performance and using environment) |
| Writing distance | 0～5m (According to the label performance and using environment) |
| Working Environment |  |
| Working temperature | -30～+70°C |
| Storage temperature | -40～+85°C |
| Humidity | 10%～95% RH Without condensation |
| Support |  |
| Document | Demo，API，Developing guide |